冷凍組織/細胞免疫螢光染色步驟

- 材料

- 耗材：22×22mm或24×50的蓋玻片(厚度No.1.5；0.17mm)、76×26mm的載玻片(silane or poly-L lysine coated)、微量吸管(10μl、200μl、1000μl)、1.5ml微量離心管、15ml離心管、10ml吸管、25ml吸管、鑷子、染缸、保濕暗盒、擦手紙、拭鏡紙、封口膜(Parafilm)、冰桶及離心管架、指甲油。

- 試劑(廠牌僅供參考)：

- 10X PBS(Merck；6505CN)

- antibody diluent(Invitrogen；003118) or 1% BSA diluted with 1X PBS

- blocking buffer：CAS-block (Invitrogen；008120) or 1% BSA diluted with 1X PBS or 10% serum from species from which the secondary antibody was raised in

- DAPI(Invitrogen；D1306)

- TritonX-100 solution(10% in H2O)( BioVision；2104-100)

- Prolong Gold Antifade Reagent (Invitrogen；P36930)

- Formaldehyde Solution, 10% (w/v) in Aqueous Phosphate Buffer (10%NBF) (Marcon；H121-08)

- Primary and Secondary antibodies(Invitrogen；Alexa-Fluor Dye series )

- 染色步驟-英文版

- Fix cells or cryostat sections in 10% Neutral buffered formalin(10%NBF) for 15min at room temperature.

- Wash with 1XPBS for 3 times, 5 min each at room temperature.

- Permeabilize cells or cryostat sections in 0.1% TritonX-100 for 15 min at room temperature.

- Wash with 1XPBS for 3 times, 5 min each at room temperature.

- To block unspecific binding of antibodies, block cells or cryostat sections with 1% BSA in PBS for 30min in a humidified chamber at room temperature.

** alternative blocking solution is 10% serum from species from which the secondary antibody was raised in

- Incubate cells or cryostat sections in diluted primary antibody in 1% BSA in PBS in a humidified chamber overnight at 4°C.

- Wash with 1XPBS for 3 times, 5 min each at room temperature.

- Incubate cells or cryostat sections in diluted secondary antibody in 1% BSA in PBS in a humidified chamber for 1hr at room temperature.

**prevent from light from this step

- Wash with 1XPBS for 3 times, 5 min each at room temperature.

- Counter stain cells or cryostat sections with 1ug/ml DAPI or Hoechst for 15 min at room temperature.

- Wash with 1XPBS for 3 times, 5 min each at room temperature.

- Mount coverslip with a drop of mounting medium that provides additional protection for fluorescent dyes or proteins form bleaching.

- Seal coverslip with nail polish to prevent movement if necessary

- 染色步驟-中文版

- 以10% 福馬林於室溫固定冷凍組織切片或細胞10分鐘。

- 以1XPBS清洗三次，每次五分鐘。

- 以0.1% TritonX-100於室溫下將凍組織切片或細胞打洞15分鐘。

- 以1XPBS清洗三次，每次五分鐘。

- 於保濕暗盒中以1% BSA室溫blocking 30分鐘以減少抗體非特異性結合。

**也可用二級抗體宿主之血清進行blocking。例如：二級抗體是Goat anti Mouse IgG時，可用Goat血清進行blocking。

- 加入以1% BSA稀釋好的一級抗體稀釋液，於保濕暗盒中以4℃與冷凍組織切片或細胞作用隔夜。

- 以1XPBS清洗三次，每次五分鐘。

- 加入以1% BSA稀釋好的二級抗體稀釋液，於保濕暗盒中與冷凍組織切片或細胞室溫作用1小時。

**此步驟開始均需避光。

- 以1XPBS清洗三次，每次五分鐘。

- 以1ug/ml DAPI或Hochest於室溫下作用15分鐘以進行細胞核染色。

- 以1XPBS清洗三次，每次五分鐘。

- 以含螢光保護劑的封片膠進行封片以降低螢光衰退。

- 於封片膠乾後，視需要以指甲油固定玻片四周，以避免日後玻片滑動脫落。

- 染色操作注意事項

  - 全程應避免細胞或組織乾掉，乾掉的樣品將導致非特異性背景訊號。

  - 每次染色均建議同時進行陽性與陰性對照組(secondary antibody only或Isotype control)，以評估染色過程正確或非特異性訊號之干擾。

  - 封片膠不宜過多或過少，封片後直立時蓋玻片不會輕易滑動為理想的封片膠量，過多或過少的封片膠將導致不平整的焦平面而影響影像品質，封片後不宜擠壓蓋玻片以避免樣品脫落變形或焦平面不完整。

  - 上機前應確認蓋玻片上無鹽類、封片膠等汙物以避免產生背景訊號。

  - 冷凍組織之包埋建議於1XPBS稍微沖洗後以乾淨紗布吸除多餘水分，於OCT室溫浸泡5分鐘後以乾冰或液態氮急速冷凍，以保持組織最佳型態。放置-80°C冰箱使組織慢慢凝固可能導致冰晶形成而破壞組織型態。

  - 蓋玻片務必選用厚度No.1.5(理想厚度0.17mm；厚度範圍0.16mm-0.19mm)以適用高NA鏡頭之倒立式共軛焦顯微鏡，以達理想影像品質。